ÎNTREPRINDEREA  DE  STAT  PENTRU  UTILIZAREA  SPAŢIULUI  AERIAN

ŞI  DESERVIREA  TRAFICULUI  AERIAN  „MOLDATSA”

C A I E T  DE   S A R C I N I

Modernizarea echipamentului activ a rețelei optice LAN MoldATSA

(Achiziție echipament și servicii de instalare

Modular Switch and 1U Stackable Switches)

C H I Ş I N Ă U  2020

Lista cerințelor și parametrilor tehnici la achiziționarea

Modular Switch.

Cerințe de instalare și livrare:

- Soluția propusă trebuie să fie livrată cu soft-ware necesar instalat pe echipament.

- Soluția propusă trebuie să fie livrată cu toate componentele hardware adiționale pentru cablare și plasare în rack.

- Soluția propusă trebuie să fie instalata si configurata conform cerințelor clientului (hardware installation, software configuration, fiber optic SFP+ connection, VLANs, management, monitoring, port security, STP, ACL, testing of failover).

Cerințe tehnice de exploatare:

- Dimensiunile tehnice a echipamentului trebuie să corespundă standardului 19”.

- Toate componentele hardware a soluției trebuie să fie compatibile cu rețeaua de curent electric AC120/240V 50/60Hz.

- Temperatura admisibilă de funcționare a echipamentului și componentelor acestuia sa fie in limita de la 0°C până la 40°C.

Interfaces

- Not less than 144 ports 10/100/1000 Base-T per switch included (possibility to extend up to 240 ports 10/100/1000 Base-T)

- not less than 8 ports 1/10Gbps SFP+ with SFP+SR 850nm transceivers included (possibility to extend up to 16 ports 1/10Gbps SFP+)

Resiliency and high availability:

- Management Modules 1+1

- Power supply modules protection 1+1 (minimum) AC120/240V 50/60Hz

- Fast Software Upgrade

- VRRP / HSRP

- Nonstop switching

- IEEE 802.1 w ,s Spanning Tree Protocol

- IEEE 802.3ad Link Aggregation Control Protocol (LACP)

- Hot-swappable modules;

Quality of Service (QoS)

- Advanced classifier-based QoS - based on Layer 2, 3, and 4 information, application of policies on a per-port or per-VLAN basis;

- IP service level agreements (SLA) for voice monitor quality of voice traffic using the UDP jitter and UDP jitter for VoIP tests.

- Traffic prioritization into eight priority levels mapped to eight queues;

- Bandwidth shaping :

  - port-based rate limiting, provides per-port ingress/egress enforced increased bandwidth;

  - Classifier-based rate limiting uses an access control list (ACL) to enforce increased bandwidth for ingress traffic on each port;

  - Reduced bandwidth provides per port, per-queue egress-based reduced bandwidth;

- Class of Service (CoS):

- Sets the IEEE 802.1p priority tag based on IP address, IP Type of Service (ToS), Layer 3 protocol, TCP/UDP port number, source port, and DiffServ.

Management

Connection methods: CLI, GUI, SNMP MIB, Management software, REST API

Out-of-band management: console port

Dual flash images for independent primary and secondary operating system files for backup while upgrading.

Link Connection monitoring:

Unidirectional Link Detection (UDLD) support

support UDLD and DLDP protocols to monitor a cable between two switches and shut down the ports on both ends if a broken link is detected, preventing network problems such as loops.

Bidirectional Forwarding Detection (BFD) support

Layer 2 switching

- VLAN support and tagging supports the IEEE 802.1Q standard and 4,094 VLANs simultaneously;

- IEEE 802.1ad Q-in-Q;

- MAC-based VLAN provides granular control and security, uses RADIUS to map a MAC address/user to specific VLANs;

- Jumbo frames up to 9200 bytes;

- MAC address table – min 64k;

- switching capacity – min 1900Gpbs

Layer 3 services

Loopback interface address, DHCP server.

Layer 3 routing

Static IP routing IPv4/IPv6, OSPFv2/v3, RIP/RIPng, BGP, Policy-based routing, 10000 IPv4/ 5000 IPv6 routes

Security

- Control Plane Policing with rate limit on control protocols to protect CPU overload from DOS attacks

- Access control lists (ACLs)

- Multiple user authentication methods:

- IEEE 802.1X users per port

- Web-based authentication(authenticates from a Web browser for clients that do not support IEEE 802.1X supplicant)

- MAC-based authentication;

- Concurrent IEEE 802.1X, Web, and MAC authentication schemes per port(switch port accepts up to 32 sessions of IEEE 802.1X, Web, and MAC authentications);

- DHCP protection

- Secure management access : delivers secure encryption of all access methods (CLI, GUI, or MIB) through SSHv2, SSL, and/or SNMPv3

- Switch CPU protection : provides automatic protection against malicious network traffic trying to shut down the switch;

- STP BPDU port protection;

- Dynamic ARP protection;

- STP Root Guard;

- Port security (allows access only to specified MAC addresses, which can be learned or specified by the administrator)

- RADIUS/TACACS+

- Secure FTP

- Private VLAN;

Convergence:

- IP multicast routing(PIM SM + PIM DM), IP multicast snooping,

- Auto VLAN configuration for voice(RADIUS VLAN: uses a standard RADIUS attribute and LLDP-MED to automatically configure a VLAN for IP phones),

- Local MAC Authentication(assigns attributes such as VLAN and QoS using locally configured profile that can be a list of MAC prefixes).

Cerințe de garanție și suport:

Soluția propusă trebuie să includă suport hardware și software pentru cel puțin 5 ani sau sa aibă “limited lifetime warranty”.

Cantitatea necesară:

- Modular Switch – 1 complet.

Lista cerințelor și parametrilor tehnici la achiziționarea

1U Stackable Switches.

Cerințe de instalare și livrare:

- Soluția propusă trebuie să fie livrată cu soft-ware necesar instalat pe echipament.

- Soluția propusă trebuie să fie livrată cu toate componentele hardware adiționale pentru cablare și plasare în rack.

- Soluția propusă trebuie să fie instalata si configurata (hardware installation, software configuration, 2 switches in stack, 9 stacks, fiber optic SFP+ connection, VLANs, management, , monitoring, port security, STP, ACL, testing of failover) .

Cerințe tehnice de exploatare:

- Dimensiunile tehnice a dulapului de distribuție trebuie să corespundă standardului 19”.

- Toate componentele hardware a soluției trebuie să fie compatibile cu rețeaua de curent electric AC120/240V 50/60Hz.

- Temperatura admisibilă de funcționare a echipamentului și componentelor acestuia sa fie in limita de la 0°C până la 40°C.

Interfaces

- Not less than 24 interfaces 10/100/1000 Base_T

- Not less than 4 SFP+ 1/10Gbps interfaces, including SFP+ 10Gbps Base-LX module (WDM-optional) SM, 1490/1550nm or 1270/1330nm, 10km.

- Not less than 1 stackable interface, including stackable cable

Resiliency and high availability:

- Power supply modules protection 1+1 AC120/240V 50/60Hz

- Fast Software Upgrade

- VRRP / HSRP

- Nonstop switching

- RRPP/REP (optional)

- IEEE 802.1 w ,s Spanning Tree Protocol

- IEEE 802.3ad Link Aggregation Control Protocol (LACP)

- Hot-swappable power supply modules

- Data-plane and management-plane stacking using dedicated module or network interfaces, stack bandwidth min 20Gbps full duplex

Quality of Service (QoS)

- Advanced classifier-based QoS - based on Layer 2, 3, and 4 information, application of policies on a per-port or per-VLAN basis;

- IP service level agreements (SLA) for voice monitor quality of voice traffic using the UDP jitter and UDP jitter for VoIP tests.

- Traffic prioritization into eight priority levels mapped to eight queues;

- Bandwidth shaping :

  - port-based rate limiting, provides per-port ingress/egress enforced increased bandwidth;

  - Classifier-based rate limiting uses an access control list (ACL) to enforce increased bandwidth for ingress traffic on each port;

  - Reduced bandwidth provides per port, per-queue egress-based reduced bandwidth;

- Class of Service (CoS):

- Sets the IEEE 802.1p priority tag based on IP address, IP Type of Service (ToS), Layer 3 protocol, TCP/UDP port number, source port, and DiffServ.

Management

Connection methods : CLI, GUI, SNMP MIB, Management software, REST API.

Out-of-band management: console port

Dual flash images for independent primary and secondary operating system files for backup while upgrading.

Link Connection monitoring:

Unidirectional Link Detection (UDLD) support

support UDLD and DLDP protocols to monitor a cable between two switches and shut down the ports on both ends if a broken link is detected, preventing network problems such as loops.

Bidirectional Forwarding Detection (BFD) support

Layer 2 switching

- VLAN support and tagging supports the IEEE 802.1Q standard and 4,094 VLANs simultaneously;

- IEEE 802.1ad Q-in-Q;

- MAC-based VLAN provides granular control and security, uses RADIUS to map a MAC address/user to specific VLANs;

- Jumbo frames up to 9200 bytes;

- MAC address table – min 32k;

- switching capacity – min 128Gpbs

- VxLAN;

Layer 3 services

Loopback interface address, DHCP server.

Layer 3 routing

Static IP routing IPv4/IPv6, OSPFv2/v3, RIP/RIPng, Policy-based routing, 2000 IPv4/ 1000 IPv6 routes

Security

- Control Plane Policing with rate limit on control protocols to protect CPU overload from DOS attacks

- Access control lists (ACLs)

- Multiple user authentication methods:

- IEEE 802.1X users per port

- Web-based authentication(authenticates from a Web browser for clients that do not support IEEE 802.1X supplicant)

- MAC-based authentication;

- Concurrent IEEE 802.1X, Web, and MAC authentication schemes per port(switch port accepts up to 32 sessions of IEEE 802.1X, Web, and MAC authentications);

- DHCP protection

- Secure management access : delivers secure encryption of all access methods (CLI, GUI, or MIB) through SSHv2, SSL, and/or SNMPv3

- Switch CPU protection : provides automatic protection against malicious network traffic trying to shut down the switch;

- STP BPDU port protection;

- Dynamic ARP protection;

- STP Root Guard;

- Port security (allows access only to specified MAC addresses, which can be learned or specified by the administrator)

- RADIUS/TACACS+

- Secure FTP

- Private VLAN;

Convergence:

- IP multicast routing(PIM SM + PIM DM), IP multicast snooping,

- Auto VLAN configuration for voice(RADIUS VLAN: uses a standard RADIUS attribute and LLDP-MED to automatically configure a VLAN for IP phones),

- Local MAC Authentication (assigns attributes such as VLAN and QoS using locally configured profile that can be a list of MAC prefixes).

Cerințe de garanție și suport:

1. Soluția propusă trebuie să includă suport hardware și software pentru cel puțin 5 ani sau sa aibă “limited lifetime warranty”.

Cantitatea necesară:

1U Stackable Switch – 18 unități + 2(rezerv)

SFP+ Module WDM 10Gbps – 18 buc. + 2 (rezerv)

Stackable Cable 3m – 18 buc. + 2 (rezerv).

Cerințe fata de candidați:

- Partener oficial al producătorului in RM.

- Dispune de cel puțin un inginer în rețele de calculatoare cu certificatul emis de producătorul echipamentului.

- Deținător de certificat ISO 9001:2015 - Sistem de management al calității.

- Deținător de certificat ISO 27001:2013 - Sistem de management al securității informației.

- Întrunirea cerințelor ”Regulamentului privind deșeurile de echipamente electrice și electronice” aprobat prin HG Nr. 212 din 07-03-2018.

Inginer coordonator SAIS DTA     		    		               Vitali Ostapov

Tel. de contact 022 502 844